## Current document focuses on the hardware design of the visual display unit of Anodiam glasses, a digitally enabled spectacle like eye wear, for displaying computational/ digital/ virtual objects, hence forth referred as ubiquitous computing object or UCO. The display unit of the Anodiam glasses will be typically designed with commercially viable and easily available elements in the market.

## The UCOs can be files, icons, buttons, images, dropdowns, IDEs, objects, QR codes, browsers, 3D object, virtual camera etc. An UCO can be both display-only and interactive through input interrupts like hand gestures (call, wave, stop, etc.), finger actions (push, touch, pluck & drag etc.), virtual stylus & forceps, physical mouse, physical keyboard etc.

## Anodiam glass display must embed the digital UCOs into real life physical objects and spaces like walls, tables, streets, trees, mid-air, sky, water, anywhere. It must adjust the parallax between an UCO displayed for the right eye vs the same UCO displayed for the left eye to create the virtual distance between the user's eyes and the UCO itself. This will provide the 3D immersive effect.

# Optical see-through waveguide glasses

A tiny microdisplay projects an image into a waveguide lens that looks like a normal clear lens, so virtual objects appear over the real world.

# Display unit block (per eye)

#### Microdisplay

- Commercially available micro-OLED; e.g., Sony’s microdisplay (includes very high pixel density parts for AR/VR)

- ~$500–$600 USD for both eyes

#### Projection optics (collimator + relay)

- Small lens stack that collimates the microdisplay so the eye can focus comfortably, typically set to a fixed focal plane like ~2meters (or compromised optical infinity).

#### Waveguide / combiner lens (transparent “screen”)

- The waveguide expands the eyebox and couples light into/out of the lens.

- Commercial example: Lumus promotes reflective waveguide products and notes ability to directly bond Rx lenses (important for prescription correction), and thin optics (e.g., “1.3 mm” class spec is cited on product pages).

- Waveguide supply chains exist (e.g., WaveOptics being used in AR glasses and acquired by Snap signals maturity of this segment).

#### Eye-side “ancillary” optics (optional)

- Used to improve eyebox, reduce distortion, and support different face geometries.

# Optical correction (prescription/ diopters)

There are 3 commercially viable approaches, we will prioritise according to the oder below:

- Bond Rx lens onto the waveguide (if the waveguide vendor supports it; Lumus explicitly mentions direct bonding of Rx lenses).

- Clip-in prescription insert behind the waveguide (common in many headsets/glasses because it keeps waveguide manufacturing simpler).

- Limited mechanical diopter adjustment (more common in bulkier headsets than slim glasses).

# Practical limitations (important)

- Field of View (FOV) in spectacle form factor is typically more limited than VR headsets (physics + waveguide constraints). (Can still build a compelling “workspace” UI, but don’t expect 120° VR-like immersion.)

- Vergence–accommodation conflict: can set stereo parallax (vergence) with left/right images like you described, but the eye’s focus (accommodation) is usually fixed at one focal plane. This is the biggest comfort limiter for close virtual objects.

Best for: “Real world + overlays” on streets, rooms, desks; all-day wearable form factor.